# Project 2

University of Nebraska–Lincoln

Department of Civil Engineering

Engineering Analysis II

Team Members:

Christopher Moore

Bhuwanraj Singh Parihar

Mohamed Adam

Daniel Kugler

Instructor: Wissam Kontar

Section Number: 150

Due Date of Assignment: February 24, 2026

Table of Contents

Scope of Work	3

Gantt Chart	4

Annotated Code:	5

Summary:	10

Introduction:	10

Methods:	10

Results & Discussion:	10

Example Scenarios:	11

Scenario 1 NDOT Class 47B	11

Scenario 2: 4000 PSI Structural Concrete (Typical ACI Mix)	13

Scenario 3: 47BD Concrete Mix	15

Scenario 4: 10000 PSI Concrete Mix	17

References:	19

# Scope of Work

Project Title: Automation of Concrete Mix Design

Team Members: Christopher Moore, Bhuwanraj Sing Parihar, Mohamed Adam, and Daniel Kugler

Submitted for review: February 24, 2026

Submitted to: Nebraska Department of Transportation (NDOT)

Project Goals: The Nebraska Department of Transportation (NDOT) has engaged this project team to automate the Excel based concrete mix design model and convert it into a Python design model. The purpose of this project is to improve the calculation, reliability, transparency, and repeatability of the model. The model provides user inputs in sequential orders and automatically calculates a final concrete mix weight chart for one cubic yard of concrete.

Project Tasks:

Task 1: Review the NDOT concrete Mix Design workbook to understand calculation flow, dependencies, and required inputs

Task 2: Identify all formulas and their parameters and provide an input-output function

Task 3: Translate the Excel function into a reusable Python function with clear instructions and names and a step-by-step user input workflow

Task 4: Verify by testing with various realistic concrete mix design scenarios

Task 5: Provide all deliverables to the client

# Gantt Chart

| PROJECT #2 - GANTT CHART |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Week 5 |  |  | Week 6 |  |  |  |  |  |  | Week 7 |  |  |  |  |  |  | Week 8 |  | Date Range |  |  |
| TASKS | 06 Feb | 07 Feb | 08 Feb | 09 Feb | 10 Feb | 11 Feb | 12 Feb | 13 Feb | 14 Feb | 15 Feb | 16 Feb | 17 Feb | 18 Feb | 19 Feb | 20 Feb | 21 Feb | 22 Feb | 23 Feb | 24 Feb | Start Date | End Date | Duration |
|  | Fri | Sat | Sun | Mon | Tues | Wed | Thur | Fri | Sat | Sun | Mon | Tues | Wed | Thur | Fri | Sat | Sun | Mon | Tues |  |  |  |
| Data Processing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 06 Feb 2026 | 06 Feb 2026 | 1 |
| Scope of Work |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 07 Feb 2026 | 10 Feb 2026 | 4 |
| Writing Code |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 06 Feb 2026 | 20 Feb 2026 | 15 |
| GitHub |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 21 Feb 2026 | 24 Feb 2026 | 4 |
| Annotated Code Document |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 19 Feb 2026 | 24 Feb 2026 | 6 |
| Final Report |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 21 Feb 2026 | 24 Feb 2026 | 4 |
| Flow Diagram |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 19 Feb 2026 | 20 Feb 2026 | 2 |

Table 1. Gantt Chart for Project 2

Progress

25

50

75

Completed

Table 2. Color coated legend to define project progress

# Annotated Code:

Table 3. Annotation of each code used in project 2

| Python Code | Annotation(s) |
| --- | --- |
| import pandas as pd import numpy as np | These import common Python libraries needed to run the code |
| project_name = "Project #2: Automation of NDOT Concrete Mix Design" client = "Nebraska Department of Transportation (NDOT)" | Stores project and client info as variables. They’re printed later so outputs are clearly tied to the correct project |
| print(f'This project is designed for: {client}') | Prints the client name using an f-string. Demonstrates formatted output in Python |
| my_number = 7.341212 print(f"{my_number:8.2f}") | Demonstrates numeric formatting. Rounds a number to two decimal places. Used later for clean output formatting |
| cubic_yard_ft3 = 27 unit_weight_water = 62.4 | Defines fixed constants for the entire code. One cubic yard is 27 cubic feet and the unit weight of water is 62.4 lb per cubic feet. Since its used repeatedly throughout code defining it once will make things simpler |
| print(f"defined constant: 1 cubic yard = {cubic_yard_ft3} cubic feet") print(f"Unit weight of water = {unit_weight_water} lb per cubic feet") | Prints the defined constants to confirm the values |
| def calculate_water_weight_Q(cement_A, fly_ash_B, silica_fume_C, other_SCM_D, wc_ratio_E): total_cementious = cement_A + fly_ash_B + silica_fume_C + other_SCM_D water_weight_Q = total_cementious * wc_ratio_E return water_weight_Q | Calculates the total water weight for the mix. First it lists all the materials needed. Then it defines total_cementious as the sum of A, B, C, and D. then it multiples itself by E to achieve Q. Returning Q sends the calculated value back to the main program |
| w_weight_1 = calculate_water_weight_Q(600, 100, 30, 70, 0.42) print(f"Water weight = {w_weight_1} lb/yd^3") | An example used for the function using random numeric values and printing the calculated water weight |
| cement_A = 600 fly_ash_B = 100 silica_fume_C = 30 other_SCM_D = 70 wc_ratio_E = 0.42 | Demonstrates using variables as inputs. |
| w_weight_2 = calculate_water_weight_Q(cement_A, fly_ash_B, silica_fume_C, other_SCM_D, wc_ratio_E) print(f"water weight = {w_weight_2} lb/yd^3") | Both methods give the same output when ran |
| def calculate_volume_cement_R(cement_A, sg_cement_J): volume_R = cement_A / (sg_cement_J * unit_weight_water) return volume_R # Example input values for demonstration cement_A = 600 sg_cement_J = 3.5 # Use them in the function R = calculate_volume_cement_R(cement_A, sg_cement_J) # Print print(f"cement volume (R): {R:.3f} ft^3") | Calculates all the different material volumes using the same formula of volume = weight divided by (specific gravity * unit weight of water) |
| def calculate_volume_fly_ash_S(fly_ash_B, sg_fly_ash_k): return fly_ash_B / (sg_fly_ash_k * unit_weight_water) def calculate_volume_silica_fume_T(silica_fume_C, sg_silica_fume_L): return silica_fume_C / (sg_silica_fume_L * unit_weight_water) def calculate_volume_SCM_U(other_SCM_D, sg_other_SCM_M): return other_SCM_D / (sg_other_SCM_M * unit_weight_water) | Structurally the same as the previous volume function |
| def calculate_air_volume_V(air_content_F): volume_V = (air_content_F /100) * cubic_yard_ft3 return volume_V def calculate_water_volume_W(weight_water_Q): volume_W = water_weight_Q / unit_weight_water return volume_W | Calculates the air and water volumes. |
| def calculate_total_aggregate_volume_X(volume_R, volume_S, volume_T, volume_U, volume_V, volume_W): volume_X = (cubic_yard_ft3 - volume_R - volume_S - volume_T - volume_U - volume_V - volume_W) return volume_X | Calculates the remaining volume available for aggregates. Subtracts all the cementitious, air, and water volumes from 27 cubic feet. Each function uses the aggregate percentage, specific gravity, and converts the volume to weight |
| def calculate_fine_aggregate_weight_Y(percent_fine_G, sg_fine_N, volume_X): weight_Y = unit_weight_water * (percent_fine_G / 100) * sg_fine_N * volume_X return weight_Y def calculate_coarse_aggregate_Z(percent_coarse_H, sg_coarse_O, volume_X): return unit_weight_water * (percent_coarse_H / 100) * sg_coarse_O * volume_X def calculate_other_aggregate_AA(percent_other_I, sg_other_P, volume_X): return unit_weight_water * (percent_other_I / 100) * sg_other_P * volume_X | Structurally the same as the previous aggregate volume function |
| # Scenario 1 – Standard Pavement Mix project_no = 101 concrete_class = "47B" cement_A = 600 fly_ash_B = 100 silica_fume_C = 30 other_SCM_D = 70 water_cement_ratio_E = 0.42 air_content_F = 6 percent_fine_G = 45 percent_coarse_H = 50 percent_other_I = 5 sg_cement_J = 3.15 sg_fly_ash_K = 2.30 sg_silica_fume_L = 2.20 sg_other_SCM_M = 2.60 sg_fine_N = 2.65 sg_coarse_O = 2.70 sg_other_P = 2.60 | User input example |
| # Start with a general explanation print("\n---------------------------------------------") print(" NDOT Concrete Mix Design – Weight Summary") print(" (1 Cubic Yard of Concrete)") print("---------------------------------------------") | print() outputs text to the console. \n inserts a blank line before the first divider. The dashed lines separate sections |
| # Project and mix details print(f"Project Number: {project_no}") print(f"Class of Concrete: {concrete_class}") print("---------------------------------------------") | f”” is an f-string. {project_no} and {concrete_class} insert variable values. This identifies info for the mix. |
| # Cementitious material outputs print(f"Cement (A): {cement_A:8.1f} lb") print(f"Fly Ash (B): {fly_ash_B:8.1f} lb") print(f"Silica Fume (C): {silica_fume_C:8.1f} lb") print(f"Other SCM (D): {other_SCM_D:8.1f} lb") print("---------------------------------------------") | These print cementious material weights. The :8.1f means that the total width is 8 characters with 1 decimal place and a fixed point format. This keeps the numbers in columns. Lb means in pounds |
| # Aggregate outputs print(f"Fine Aggregate (Y): {Y:8.0f} lb") print(f"Coarse Aggregate (Z): {Z:8.0f} lb") print(f"Other Aggregate (AA): {AA:8.0f} lb") print("---------------------------------------------") | Prints aggregate weights. :8.0f means its an 8 character width with 0 decimal places and rounded to the whole number |
| # Water output print(f"Water (Q): {Q:8.0f} lb") | Prints total batch water |
| # End it with a note print("---------------------------------------------") print("End of Mix Design Summary") | Final divider line |
| 2nd block is the same so no need to repeat. When running every piece of code and inputting, you will get a final chart that displays everything |  |

# Summary:

### Introduction:

The project focused on making an automated concrete mix design program, which was created from an excel sheet provided by the Nebraska Department of Transportation. We took the provided excel sheet and used it to make a structured program using python to calculate the requested formulas to a mixed concrete design.

### Methods:

To complete the requested proposal, the excel formulas were rewritten as python functions and by using a step-by-step user input prompt to help guide the user through the mix design process. The program itself generates a finalized weight chart for one cubic yard of concrete. At the end of the program, four realistic concrete mix scenarios were tested to showcase to the client that the code works as intended.

### Results & Discussion:

The tested results showed that the program produced an expected output based on the variables imputed. The differences between the four tested mixed designs matched expectations, which are based on realistically used mix design scenarios. Overall, the automated concrete mix design program took the original spreadsheet and provides a proposed code that introduces efficiency through user imputed prompts.

# Example Scenarios:

### Scenario 1 NDOT Class 47B

Table 4. Scenario 1 NDOT Class 47B

| Parameter | Value |
| --- | --- |
| project_no | 101 |
| concrete_class | "47B" |
| cement_A | 580 lb |
| fly_ash_B | 80 lb |
| silica_fume_C | 0 lb |
| other_SCM_D | 0 lb |
| water_cement_ratio_E | 0.44 |
| air_content_F | 7 |
| percent_fine_G | 45 |
| percent_coarse_H | 50 |
| percent_other_I | 5 |
| sg_cement_J | 3.15 |
| sg_fly_ash_K | 2.30 |
| sg_silica_fume_L | 2.20 |
| sg_other_SCM_M | 2.60 |
| sg_fine_N | 2.65 |
| sg_coarse_O | 2.70 |
| sg_other_P | 2.60 |

Table 5. Created by using the proposed code and scenario 1

---------------------------------------------

NDOT Concrete Mix Design – Weight Summary

(1 Cubic Yard of Concrete)

---------------------------------------------

Project Number:       101

Class of Concrete:    47B

---------------------------------------------

Cement (A):               580.0 lb

Fly Ash (B):               80.0 lb

Silica Fume (C):            0.0 lb

Other SCM (D):              0.0 lb

---------------------------------------------

Fine Aggregate (Y):        1207 lb

Coarse Aggregate (Z):      1366 lb

Other Aggregate (AA):       132 lb

---------------------------------------------

Water (Q):                  290 lb

---------------------------------------------

End of Mix Design Summary

Scenario 1 referenced from:

https://dot.nebraska.gov/media/jjwpyezr/pavdesignmanual.pdf (page 63 table)

### Scenario 2: 4000 PSI Structural Concrete (Typical ACI Mix)

Table 6. Scenario 2 (Typical ACI Mix)

| Parameter | Value |
| --- | --- |
| project_no | 102 |
| concrete_class | "4000psi Standard" |
| cement_A | 650 lb |
| fly_ash_B | 50 lb |
| silica_fume_C | 0 lb |
| other_SCM_D | 50 lb |
| water_cement_ratio_E | 0.42 |
| air_content_F | 6 |
| percent_fine_G | 46 |
| percent_coarse_H | 49 |
| percent_other_I | 5 |
| sg_cement_J | 3.15 |
| sg_fly_ash_K | 2.30 |
| sg_silica_fume_L | 2.20 |
| sg_other_SCM_M | 2.60 |
| sg_fine_N | 2.65 |
| sg_coarse_O | 2.70 |
| sg_other_P | 2.60 |

Table 7. Created by using the proposed code and scenario 2

---------------------------------------------

NDOT Concrete Mix Design – Weight Summary

(1 Cubic Yard of Concrete)

---------------------------------------------

Project Number:       101

Class of Concrete:    47B

---------------------------------------------

Cement (A):               650.0 lb

Fly Ash (B):               50.0 lb

Silica Fume (C):            0.0 lb

Other SCM (D):              0.0 lb

---------------------------------------------

Fine Aggregate (Y):        1219 lb

Coarse Aggregate (Z):      1324 lb

Other Aggregate (AA):       130 lb

---------------------------------------------

Water (Q):                  315 lb

---------------------------------------------

End of Mix Design Summary

Scenario 2 referenced from:

https://static1.squarespace.com/static/59c91fb8f7e0ab097112fbc4/t/5b5b22a81ae6cf14d8dcd716/1532699305261/Mix+Designs-COMBINED+072718.pdf

### Scenario 3: 47BD Concrete Mix

Table 8. Scenario 3 – 47BD Concrete Mix

project_no = 103 concrete_class = "47BD" cement_A = 658 fly_ash_B = 0 silica_fume_C = 0 other_SCM_D = 0 water_cement_ratio_E = 0.45 air_content_F = 7 percent_fine_G = 45 percent_coarse_H = 50 percent_other_I = 5 sg_cement_J = 3.15 sg_fly_ash_K = 2.30 sg_silica_fume_L = 2.20 sg_other_SCM_M = 2.60 sg_fine_N = 2.65 sg_coarse_O = 2.70 sg_other_P = 2.60

Table 9. Created by using the proposed code and scenario 3

NDOT Concrete Mix Design – Weight Summary

(1 Cubic Yard of Concrete)

---------------------------------------------

Project Number:       101

Class of Concrete:    47BD

---------------------------------------------

Cement (A):               658.0 lb

Fly Ash (B):                0.0 lb

Silica Fume (C):            0.0 lb

Other SCM (D):              0.0 lb

---------------------------------------------

Fine Aggregate (Y):        1219 lb

Coarse Aggregate (Z):      1380 lb

Other Aggregate (AA):       133 lb

---------------------------------------------

Water (Q):                  296 lb

---------------------------------------------

End of Mix Design Summary

Scenario 3 referenced from:

https://dot.nebraska.gov/media/jjwpyezr/pavdesignmanual.pdf (page 63 table)

### Scenario 4: 10000 PSI Concrete Mix

Table 10. Scenario 4 – 10,000 PSI Concrete

project_no = 104 concrete_class = "NDOT Structural – Special Provision" cement_A = 893 fly_ash_B = 0 silica_fume_C = 63 other_SCM_D = 50 water_cement_ratio_E = 0.27 air_content_F = 6.5 percent_fine_G = 45 percent_coarse_H = 50 percent_other_I = 5 sg_cement_J = 3.15 sg_fly_ash_K = 2.30 sg_silica_fume_L = 2.20 sg_other_SCM_M = 2.60 sg_fine_N = 2.65 sg_coarse_O = 2.70 sg_other_P = 2.60

Table 11. Created by using the proposed code and scenario 3

NDOT Concrete Mix Design – Weight Summary

(1 Cubic Yard of Concrete)

---------------------------------------------

Project Number:       101

Class of Concrete:    NDOT Structural

---------------------------------------------

Cement (A):               893.0 lb

Fly Ash (B):                0.0 lb

Silica Fume (C):           63.0 lb

Other SCM (D):              0.0 lb

---------------------------------------------

Fine Aggregate (Y):        1083 lb

Coarse Aggregate (Z):      1226 lb

Other Aggregate (AA):       118 lb

---------------------------------------------

Water (Q):                  272 lb

---------------------------------------------

End of Mix Design Summary

Scenario 3 referenced from:

https://static1.squarespace.com/static/59c91fb8f7e0ab097112fbc4/t/5b5b22a81ae6cf14d8dcd716/1532699305261/Mix+Designs-COMBINED+072718.pdf

## References:

Nebraska Department of Transportation. NDOT Concrete Mix Excel.

https://dot.nebraska.gov/media/jp3paote/mix-design-submittal.xlsx (accessed February 2026).

Nebraska Department of Transportation. Optimized Aggregate Gradations for Portland Cement Concrete Mix Designs Evaluation: Final Report (Report P-336). Lincoln, NE: Nebraska Department of Transportation.

https://dot.nebraska.gov/media/3isfdv45/final-report-p336.pdf (accessed February 2026).

Nebraska Department of Transportation. Standard Specifications for Highway Construction. Lincoln, NE: Nebraska Department of Transportation, 2017.

https://dot.nebraska.gov/media/g4qp4y0d/2017-specbook.pdf (accessed February 2026).

Reference material for our 4 tested scenarios:

Scenario 1  Nebraska Department of Transportation. 2018. Pavement Design Manual. Lincoln, NE: Nebraska Department of Transportation. https://dot.nebraska.gov/media/jjwpyezr/pavdesignmanual.pdf

Scenario 2  Nebraska Department of Transportation. 2018. Mix Designs – Combined. https://static1.squarespace.com/static/59c91fb8f7e0ab097112fbc4/t/5b5b22a81ae6cf14d8dcd716/1532699305261/Mix+Designs-COMBINED+072718.pdf

Scenario 3  Nebraska Department of Transportation. 2018. Pavement Design Manual. Lincoln, NE: Nebraska Department of Transportation. https://dot.nebraska.gov/media/jjwpyezr/pavdesignmanual.pdf

Scenario 4  Nebraska Department of Transportation. 2018. Mix Designs – Combined. https://static1.squarespace.com/static/59c91fb8f7e0ab097112fbc4/t/5b5b22a81ae6cf14d8dcd716/1532699305261/Mix+Designs-COMBINED+072718.pdf