Scenario 1 NDOT Class 47B Reference: https://dot.nebraska.gov/media/jjwpyezr/pavdesignmanual.pdf (page 63 table)

| Parameter | Value |
| --- | --- |
| project_no | 101 |
| concrete_class | "47B" |
| cement_A | 580 lb |
| fly_ash_B | 80 lb |
| silica_fume_C | 0 lb |
| other_SCM_D | 0 lb |
| water_cement_ratio_E | 0.44 |
| air_content_F | 7 |
| percent_fine_G | 45 |
| percent_coarse_H | 50 |
| percent_other_I | 5 |
| sg_cement_J | 3.15 |
| sg_fly_ash_K | 2.30 |
| sg_silica_fume_L | 2.20 |
| sg_other_SCM_M | 2.60 |
| sg_fine_N | 2.65 |
| sg_coarse_O | 2.70 |
| sg_other_P | 2.60 |
| --------------------------------------------- NDOT Concrete Mix Design – Weight Summary (1 Cubic Yard of Concrete) --------------------------------------------- Project Number: 101 Class of Concrete: 47B --------------------------------------------- Cement (A): 580.0 lb Fly Ash (B): 80.0 lb Silica Fume (C): 0.0 lb Other SCM (D): 0.0 lb --------------------------------------------- Fine Aggregate (Y): 1207 lb Coarse Aggregate (Z): 1366 lb Other Aggregate (AA): 132 lb --------------------------------------------- Water (Q): 290 lb --------------------------------------------- End of Mix Design Summary |  |

Scenario 2: 4000 PSI Structural Concrete (Typical ACI Mix) Reference: https://static1.squarespace.com/static/59c91fb8f7e0ab097112fbc4/t/5b5b22a81ae6cf14d8dcd716/1532699305261/Mix+Designs-COMBINED+072718.pdf

| Parameter | Value |
| --- | --- |
| project_no | 102 |
| concrete_class | "4000psi Standard" |
| cement_A | 650 lb |
| fly_ash_B | 50 lb |
| silica_fume_C | 0 lb |
| other_SCM_D | 50 lb |
| water_cement_ratio_E | 0.42 |
| air_content_F | 6 |
| percent_fine_G | 46 |
| percent_coarse_H | 49 |
| percent_other_I | 5 |
| sg_cement_J | 3.15 |
| sg_fly_ash_K | 2.30 |
| sg_silica_fume_L | 2.20 |
| sg_other_SCM_M | 2.60 |
| sg_fine_N | 2.65 |
| sg_coarse_O | 2.70 |
| sg_other_P | 2.60 |

---------------------------------------------

NDOT Concrete Mix Design – Weight Summary

(1 Cubic Yard of Concrete)

---------------------------------------------

Project Number:       101

Class of Concrete:    47B

---------------------------------------------

Cement (A):               650.0 lb

Fly Ash (B):               50.0 lb

Silica Fume (C):            0.0 lb

Other SCM (D):              0.0 lb

---------------------------------------------

Fine Aggregate (Y):        1219 lb

Coarse Aggregate (Z):      1324 lb

Other Aggregate (AA):       130 lb

---------------------------------------------

Water (Q):                  315 lb

---------------------------------------------

End of Mix Design Summary

Scenario 3: 47BD Concrete Mix https://dot.nebraska.gov/media/jjwpyezr/pavdesignmanual.pdf (page 63 table)
project_no = 103
concrete_class = "47BD"
cement_A = 658 
fly_ash_B = 0 
silica_fume_C = 0
other_SCM_D = 0
water_cement_ratio_E = 0.45 
air_content_F = 7 
percent_fine_G = 45 
percent_coarse_H = 50 
percent_other_I = 5

sg_cement_J = 3.15
sg_fly_ash_K = 2.30
sg_silica_fume_L = 2.20
sg_other_SCM_M = 2.60
sg_fine_N = 2.65
sg_coarse_O = 2.70
sg_other_P = 2.60

---------------------------------------------

NDOT Concrete Mix Design – Weight Summary

(1 Cubic Yard of Concrete)

---------------------------------------------

Project Number:       101

Class of Concrete:    47BD

---------------------------------------------

Cement (A):               658.0 lb

Fly Ash (B):                0.0 lb

Silica Fume (C):            0.0 lb

Other SCM (D):              0.0 lb

---------------------------------------------

Fine Aggregate (Y):        1219 lb

Coarse Aggregate (Z):      1380 lb

Other Aggregate (AA):       133 lb

---------------------------------------------

Water (Q):                  296 lb

---------------------------------------------

End of Mix Design Summary

Scenario 4: 10000 PSI Concrete Mix

https://static1.squarespace.com/static/59c91fb8f7e0ab097112fbc4/t/5b5b22a81ae6cf14d8dcd716/1532699305261/Mix+Designs-COMBINED+072718.pdf

project_no = 104

concrete_class = "NDOT Structural – Special Provision"

cement_A = 893

fly_ash_B = 0

silica_fume_C = 63

other_SCM_D = 50

water_cement_ratio_E = 0.27

air_content_F = 6.5

percent_fine_G = 45

percent_coarse_H = 50

percent_other_I = 5

sg_cement_J = 3.15

sg_fly_ash_K = 2.30

sg_silica_fume_L = 2.20

sg_other_SCM_M = 2.60

sg_fine_N = 2.65

sg_coarse_O = 2.70

sg_other_P = 2.60

---------------------------------------------

NDOT Concrete Mix Design – Weight Summary

(1 Cubic Yard of Concrete)

---------------------------------------------

Project Number:       101

Class of Concrete:    NDOT Structural

---------------------------------------------

Cement (A):               893.0 lb

Fly Ash (B):                0.0 lb

Silica Fume (C):           63.0 lb

Other SCM (D):              0.0 lb

---------------------------------------------

Fine Aggregate (Y):        1083 lb

Coarse Aggregate (Z):      1226 lb

Other Aggregate (AA):       118 lb

---------------------------------------------

Water (Q):                  272 lb

---------------------------------------------

End of Mix Design Summary