# Introduction/Background

Air Pollution is a significant environmental and public health concern that can cause various cardiovascular and respiratory diseases. We used PurpleAir’s sensors that provide real time air quality data by measuring pollutants such as volatile organic compounds (VOCs), particulate matter smaller than 2.5 microns (PM2.5), and those smaller than 10 microns (PM10). The goal of this project is to analyze the data and help identify pollution trends using environmental conditions such as humidity, temperature, and altitude. These conditions affect pollution levels and by using the United States Environmental Protection Agency (EPA) Air Quality Index (AQI), we can figure out a way to mitigate the pollution.

# Methods

The raw CSV file containing the data was imported into Python using the pandas library. It was then organized into a data frame and then cleaned for analysis. New columns were created for humidity and temperature using the client defined ranges as Low (<50%), High (50%-80%), and Very High (>80%) and Below Freezing (<32 F), Cool (32-50F), Warm (50-70F), and Hot (>70F). The data was then grouped using the groupby function and from this we calculated the mean and median pollution values for different environmental categories at each sensor location. To identify the hotspots, we reanked sensor by their average VOC, PM2.5, and PM10 levels. The idmax function determined the exact date and location where the max pollution values occurred. The AQI health risk events were found by filtering the rows where PM2.5 values were greater than or equal to 35.5 µg/m³ and PM10 values were greater than or equal to 155 µg/m³. These values are considered unhealthy for Sensitive groups per the EPA. We sued pd.cut to group the sensors by altitude to see whether elevation influenced the pollution levels.

# Results

The Very High humidity range showed the highest average PM2.5 and PM10 levels. This suggests that higher moisture content in the air contributes to increased particulate pollution because of the reduced dispersion of particles. We found that the Below Freezing temperature range showed the highest average pollution levels. The warm air contributes to pollutant accumulation and chemical reactions in the atmosphere. These 5 sensors were identified as the most consistent pollution hotspots based on their mean pollution levels: Broken Bow, #16 Richardson County Courthouse, #18 Southeast District Health Department in Tecumseh, NCDHD O’Neill #11, and Swnphd-mccook. These locations probably are located near areas with lots of traffic, industrialization, or poor air circulation which can explain provide explanations for such pollution. The highest pollution readings for VOC, PM2.5, and PM10 were these: All occurred at Richardson County Courthouse on 2/18/25 and a value of 3784.68.

Multiple dates were found where the pollution was exceeding the AQI limits for sensitive groups. The sensor with the highest number of exceedances was Broken Bow with 176. Levels like these cause health risks for children, the elderly, and those with heart/lung conditions. When grouped by elevation, the Medium Elevation group showed the highest average pollution levels. The evidence suggests that topography may influence how pollutants accumulate or disperse.

# Conclusion

This analysis identified the clear relationships between the environment and air pollution. Using data from AirPurple a good starting point to resolving these issues have been created and will provide a safe place to live for the general public.